Run MProg3.0 from Start Menu

Exit any KMotion Programs

Load KFLOP.ept from USB Driver directory

Run CCS

Connect JTAG

Plug in USB + BOTH Jumpers

Quickly Connect

Reset

Gel SetEMIF

Load Program (KFLOP3XX.out)

Run

Wait for Ready to use

In MProg – SCAN (Magnify glass)

Program Lightning bolt

Exit

Unplug USB cable – Plug back in

Re-connect CCS

Reset – Gel EMIF – Reload program - run

KFLOP should be detected – Auto - Next

Navigate to USB Driver directory if required

Run KFLOP3xx\KMotion.exe

CProgram – c:\KmotionSrc\C Programs\Set serial number Prog Boot FLASH KFLOP2.c

Set Serial number – then RUN

Config FLASh – New Version = KFLOP3xx\DSP_KFLOP\DSPKFLOP.out

Unplug USB – Unplug JTAG -  Plugin SelfTest Cable – Plugin USB

Run - SelfTestKFLOP.c

Unplug – Mark Serial number Size 1x1 KF30xx

Remove jumper