#include "KMotionDef.h"

#define ESTOP 172  //define ESTOP bit (normally low in my case)

#define TMP 10 // which spare persist to use to transfer data

#include "KflopToKMotionCNCFunctions.c"

// Defines axis 0,1 and 2 as simple steppers

// enables them

// sets them as an xyz coordinate system for GCode

// added stop router and raise Z for Estop

int main()

{

KStepPresent=TRUE; //Let's Kflop know a Kstep is present

Delay_sec(0.001); //Give system time to update

SetBitDirection(45,1); // Set bit 45 to an output

SetBit(45); // set bit to a 1 to activate Kstep

SetBitDirection(0,1); // set bit 0 to an output (router motor on/off)

ClearBit(0); // set bit to a 1 (which turns router off)

FPGA(STEP_PULSE_LENGTH_ADD) = 63 - 0x80; // set polarity and pulse length to 4us

ch0->InputMode=NO_INPUT_MODE;

ch0->OutputMode=STEP_DIR_MODE;

ch0->Vel=4000;

ch0->Accel=400000;

ch0->Jerk=4e+006;

ch0->P=0;

ch0->I=0.01;

ch0->D=0;

ch0->FFAccel=0;

ch0->FFVel=0;

ch0->MaxI=200;

ch0->MaxErr=1e+006;

ch0->MaxOutput=200;

ch0->DeadBandGain=1;

ch0->DeadBandRange=0;

ch0->InputChan0=0;

ch0->InputChan1=0;

ch0->OutputChan0=8;

ch0->OutputChan1=0;

ch0->MasterAxis=-1;

ch0->LimitSwitchOptions=0x100;

ch0->LimitSwitchNegBit=0;

ch0->LimitSwitchPosBit=0;

ch0->SoftLimitPos=1e+030;

ch0->SoftLimitNeg=-1e+030;

ch0->InputGain0=1;

ch0->InputGain1=1;

ch0->InputOffset0=0;

ch0->InputOffset1=0;

ch0->OutputGain=1;

ch0->OutputOffset=0;

ch0->SlaveGain=1;

ch0->BacklashMode=BACKLASH_OFF;

ch0->BacklashAmount=0;

ch0->BacklashRate=0;

ch0->invDistPerCycle=1;

ch0->Lead=0;

ch0->MaxFollowingError=1000000000;

ch0->StepperAmplitude=20;

ch0->iir[0].B0=1;

ch0->iir[0].B1=0;

ch0->iir[0].B2=0;

ch0->iir[0].A1=0;

ch0->iir[0].A2=0;

ch0->iir[1].B0=1;

ch0->iir[1].B1=0;

ch0->iir[1].B2=0;

ch0->iir[1].A1=0;

ch0->iir[1].A2=0;

ch0->iir[2].B0=0.000769;

ch0->iir[2].B1=0.001538;

ch0->iir[2].B2=0.000769;

ch0->iir[2].A1=1.92076;

ch0->iir[2].A2=-0.923833;

EnableAxisDest(0,0);

ch1->InputMode=NO_INPUT_MODE;

ch1->OutputMode=STEP_DIR_MODE;

ch1->Vel=80000;

ch1->Accel=300000;

ch1->Jerk=500000;

ch1->P=0;

ch1->I=0.01;

ch1->D=0;

ch1->FFAccel=0;

ch1->FFVel=0;

ch1->MaxI=200;

ch1->MaxErr=1e+006;

ch1->MaxOutput=200;

ch1->DeadBandGain=0;

ch1->DeadBandRange=0;

ch1->InputChan0=1;

ch1->InputChan1=1;

ch1->OutputChan0=9;

ch1->OutputChan1=0;

ch1->MasterAxis=-1;

ch1->LimitSwitchOptions=0x100;

ch1->LimitSwitchNegBit=0;

ch1->LimitSwitchPosBit=0;

ch1->SoftLimitPos=1e+030;

ch1->SoftLimitNeg=-1e+030;

ch1->InputGain0=1;

ch1->InputGain1=1;

ch1->InputOffset0=0;

ch1->InputOffset1=0;

ch1->OutputGain=1;

ch1->OutputOffset=0;

ch1->SlaveGain=1;

ch1->BacklashMode=BACKLASH_OFF;

ch1->BacklashAmount=0;

ch1->BacklashRate=0;

ch1->invDistPerCycle=1;

ch1->Lead=0;

ch1->MaxFollowingError=1000000000;

ch1->StepperAmplitude=20;

ch1->iir[0].B0=1;

ch1->iir[0].B1=0;

ch1->iir[0].B2=0;

ch1->iir[0].A1=0;

ch1->iir[0].A2=0;

ch1->iir[1].B0=1;

ch1->iir[1].B1=0;

ch1->iir[1].B2=0;

ch1->iir[1].A1=0;

ch1->iir[1].A2=0;

ch1->iir[2].B0=1;

ch1->iir[2].B1=0;

ch1->iir[2].B2=0;

ch1->iir[2].A1=0;

ch1->iir[2].A2=0;

EnableAxisDest(1,0);

ch2->InputMode=NO_INPUT_MODE;

ch2->OutputMode=STEP_DIR_MODE;

ch2->Vel=80000;

ch2->Accel=300000;

ch2->Jerk=500000;

ch2->P=0;

ch2->I=0.01;

ch2->D=0;

ch2->FFAccel=0;

ch2->FFVel=0;

ch2->MaxI=200;

ch2->MaxErr=1e+006;

ch2->MaxOutput=200;

ch2->DeadBandGain=0;

ch2->DeadBandRange=0;

ch2->InputChan0=2;

ch2->InputChan1=2;

ch2->OutputChan0=10;

ch2->OutputChan1=0;

ch2->MasterAxis=1;

ch2->LimitSwitchOptions=0x100;

ch2->LimitSwitchNegBit=0;

ch2->LimitSwitchPosBit=0;

ch2->SoftLimitPos=1e+030;

ch2->SoftLimitNeg=-1e+030;

ch2->InputGain0=1;

ch2->InputGain1=1;

ch2->InputOffset0=0;

ch2->InputOffset1=0;

ch2->OutputGain=1;

ch2->OutputOffset=0;

ch2->SlaveGain=1;

ch2->BacklashMode=BACKLASH_OFF;

ch2->BacklashAmount=0;

ch2->BacklashRate=0;

ch2->invDistPerCycle=1;

ch2->Lead=0;

ch2->MaxFollowingError=1000000000;

ch2->StepperAmplitude=20;

ch2->iir[0].B0=1;

ch2->iir[0].B1=0;

ch2->iir[0].B2=0;

ch2->iir[0].A1=0;

ch2->iir[0].A2=0;

ch2->iir[1].B0=1;

ch2->iir[1].B1=0;

ch2->iir[1].B2=0;

ch2->iir[1].A1=0;

ch2->iir[1].A2=0;

ch2->iir[2].B0=1;

ch2->iir[2].B1=0;

ch2->iir[2].B2=0;

ch2->iir[2].A1=0;

ch2->iir[2].A2=0;

EnableAxisDest(1,0);

ch3->InputMode=NO_INPUT_MODE;

ch3->OutputMode=STEP_DIR_MODE;

ch3->Vel=40000;

ch3->Accel=400000;

ch3->Jerk=4e+006;

ch3->P=0;

ch3->I=0.01;

ch3->D=0;

ch3->FFAccel=0;

ch3->FFVel=0;

ch3->MaxI=200;

ch3->MaxErr=1e+006;

ch3->MaxOutput=200;

ch3->DeadBandGain=1;

ch3->DeadBandRange=0;

ch3->InputChan0=0;

ch3->InputChan1=0;

ch3->OutputChan0=11;

ch3->OutputChan1=0;

ch3->MasterAxis=-1;

ch3->LimitSwitchOptions=0x100;

ch3->LimitSwitchNegBit=0;

ch3->LimitSwitchPosBit=0;

ch3->SoftLimitPos=1e+009;

ch3->SoftLimitNeg=-1e+009;

ch3->InputGain0=1;

ch3->InputGain1=1;

ch3->InputOffset0=0;

ch3->InputOffset1=0;

ch3->OutputGain=1;

ch3->OutputOffset=0;

ch3->SlaveGain=1;

ch3->BacklashMode=BACKLASH_OFF;

ch3->BacklashAmount=0;

ch3->BacklashRate=0;

ch3->invDistPerCycle=1;

ch3->Lead=0;

ch3->MaxFollowingError=1000000000;

ch3->StepperAmplitude=20;

ch3->iir[0].B0=1;

ch3->iir[0].B1=0;

ch3->iir[0].B2=0;

ch3->iir[0].A1=0;

ch3->iir[0].A2=0;

ch3->iir[1].B0=1;

ch3->iir[1].B1=0;

ch3->iir[1].B2=0;

ch3->iir[1].A1=0;

ch3->iir[1].A2=0;

ch3->iir[2].B0=0.000769;

ch3->iir[2].B1=0.001538;

ch3->iir[2].B2=0.000769;

ch3->iir[2].A1=1.92081;

ch3->iir[2].A2=-0.923885;

EnableAxisDest(2,0);

DefineCoordSystem(0,1,2,-1);

//this is a continuous loop that checks the ESTOP bit and

//disables all Axis

while(ReadBit(ESTOP)!=1)	 // loop until ESTOP bit goes to specified polarity

WaitNextTimeSlice();

DisableAxis(0); //Stop X

DisableAxis(1); //Stop Y

ClearBit(0); // set bit 0 to a 0 (which turns router off)

Move(2,-1600);	// raises Z to just under soft limit

while (!CheckDone(2));  //Wait till motion stops

DisableAxis(2);	// Stop Z and survey the damage

return 0;

}